# METHOD OF STATEMENT

Date: 03rd June 2021

Description of work

To hoist up 1 unit machine to 5th floor through shutter door opening by using platform and mobile crane

Machine Spec

1 unit DMG Monoblock 65

Dim: 310 x 260 x 285 cm

Wt: 12, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBC

Time of Operation

TBC

Location of Operation

#### 39 Changi North Crescent

#### UTC Aerospace System Singapore

Equipment Use

1 x 200 ton mobile crane

1 x lorry loader

1 x platform

1 set wire sling

Shackle

Rollers

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated hoisting area

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Hoist up platform together with machine to 5th floor shutter door opening

- Stabilize platform effectively and ensure platform is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Jack up machine to 3” high and insert rollers under machine bed

- Manual pulley is slowly release to control the moving speed of machine

- Shift out machine from platform

- Position machine at the designated location

- Unload from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175