# METHOD OF STATEMENT

Date: 25th Apr 2023

Description of work

To shift out 3 unit machine from 2nd floor via cargo lift, transfer to new building, hoist up 1 unit machine to 3rd floor through shutter door opening by using platform and mobile crane and move in 2 unit machine to 3rd floor via cargo lift

Machine Spec

Via Hoisting

1 unit Nanotechnology 350FG Moore machine

Dim: 200 x 170 x 210 cm

Wt: 3, 200 kg

Via cargo lift

1 unit Kennametal Extrude Hone machine

Dim: 160 x 110 x 225 cm / Wt: 500 kg

1 unit Micro UPM machine

Dim: 110 x 85 x 175 cm / Wt: 200 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

06/05/23 (Saturday)

Time of Operation

09:45 am

Location of Operation

#### Singapore Institute of Manufacturing Technology

#### Move out via cargo: 5 Clean Tech Loop, CT2B

#### Move in via hoisting to 3rd flor: CT2A

Equipment Use

1 x 100 ton mobile crane

1-2 x lorry loader

1 x platform

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- Hoist down machine from lorry chassis onto platform

- Secured machine onto platform with strapping belt

- Hoist up platform together with machine to 3rd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert pallet truck under machine base

- Use of machine pulley to slowly release and control moving speed of machine

- Shift out machine from platform

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175