# METHOD OF STATEMENT

Date: 01st June 2020

Description of work

To hoist up 1 unit machine to 2nd floor through gate opening by using platform and lorry crane

Machine Spec

Boyke Extruder Machine

Dim: 1950 x 650 x 1922 mm

Wt: 1, 100 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

08/06/2020

Time of Operation

09:45 am

Location of Operation

#### 1 Senoko South Rd

#### Sumitomo Bakelite (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x platform

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Hoist down case from forwarder lorry

- Uncrate case

- Hoist and detach machine from base

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Hoist up platform together with machine to 2nd floor gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert pallet truck under machine base

- Shift out machine from platform

- Position machine into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175