# METHOD OF STATEMENT

Date: 09th Sep 2025

Description of work

To hoist up 3 units Bar Feeder to 6th floor by using platform and mobile crane

Bar Feeder Spec

3 units LNS Bar Feeder

Dim: 390 x 47 x 102 cm / Wt: 300 kgs each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Time of Operation

10:00 am

Location of Operation

#### 10 Tampines North Drive 4 – JTC Space @ Tampines North

#### Turntech Precision Engineering Pte Ltd #06-08/14

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x platform

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Implementation of ‘traffic control’ such as assignment of worker to act as banksman/traffic controller to assist in controlling movement of pedestrians and vehicle within the site vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated hoisting area for the lift

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Load bar feeder onto platform

- Secured bar feeder onto platform with strapping belt

- Hoist up platform together with bar feeder to 6th floor opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert pallet truck/rollers to bar feeder stand

- Shift out bar feeder from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be dispose and clear

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175