# METHOD OF STATEMENT

Date: 24th Oct 2019

Description of work

To shift out 1 unit machine from blk S ground floor, load onto lorry chassis, transfer to blk R, hoist up to 3rd floor by using platform and lorry crane

Machine Spec

Meister G3 machine

Dim: 270 x 150 x 175 cm

Wt: 2, 800 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

01/11/2019 (Friday)

Time of Operation

09:40 am

Location of Operation

#### 180 A.M.K Ave 8, Nanyang Polytechnic

From Blk S ground floor - Blk R 3rd floor

Equipment Use

1 x 50 ton mobile crane

2 x 633 lorry loader

1 x 4.5 ton forklift

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, mobile/lorry crane operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Shift out machine from blk S to loading area

- Forklift to fork up machine onto lorry chassis and transport to blk R

- Fork machine and load onto platform

- Secured machine onto platform with strapping belt

- 1 x 50 ton mobile crane to hoist up platform together with machine to 3rd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert pallet truck under machine base

- Shift out machine from platform and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175