# METHOD OF STATEMENT

Date: 19th Mar 2020

Description of work

To shift out and hoist down 8 unit machine from 5th floor to 3rd floor by using platform and mobile/lorry crane

Machine Spec

Neutron NV-500

Dim: 240 x 152 x 237 cm

Wt: 3, 230 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

26/03/2020

Time of Operation

09:30 am

Location of Operation

#### Blk 20 Woodlands Link, From #05-06 to #03-06

#### New-way Synergies Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 633 PALFIGER lorry loader

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- Hoist up platform to 5th floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift machine into platform

- Secured machine onto platform with strapping belt

- Hoist down platform together with machine from 5th floor to 3rd floor

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Shift out machine from platform

- Position machine into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd / Tel: 62824175