# METHOD OF STATEMENT

Date: 23rd Aug 2022

Description of work

1) To hoist up 2 unit machine to 9th floor through swing gate opening by using platform and mobile crane

2) To shift out 2 unit machine from 7th floor and hoist up to 9th floor

Machine Spec

2 unit new machine to 9th floor

750 CNC

Dim: 170 x 235 x 240 cm / Wt: 4, 000 kg

860 CNC

Dim: 220 x 240 x 260 cm / Wt: 6, 000 kg

From 7th floor to 9th floor

860 CNC

Dim: 220 x 240 x 260 cm / Wt: 6, 000 kg

MC560 CNC Lathe

Wt: 4, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

03/09/22 (Saturday)

Time of Operation

9:45 am

Location of Operation

#### Blk 20 Woodlands Link #07-26 & #09-34

#### Qian Cheng Engineering

Equipment Use

1 x 100 ton mobile crane

2 x lorry crane

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated hoisting area for the lift

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Hoist up platform together with machine to 9th floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the swing gate opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert pallet truck/rollers under machine base

- Shift out machine from platform

- Position machine into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175