# METHOD OF STATEMENT

Date: 13th Dec 2019

Description of work

To hoist up 2 cases of machine to 3rd floor by using platform and mobile crane

Case Spec

Case 1: 396 x 225 x 221 cm

Wt: 6, 500 g

Case 2: 340 x 227 x 257 cm

Wt: 7, 120 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

13/12/2019/2019

Time of Operation

10:00 am

Location of Operation

#### 25 Sungei Kadut St 2

Equipment Use

1 x 50 ton mobile crane

2 x lorry loader

1 x platform

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile crane at designated hoisting area

- Secured case onto platform with strapping belt

- Hoist up platform together with case to 3rd floor opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert rollers/pallet truck under case base

- Shift out case from platform

- Position case at temporary location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175