# METHOD OF STATEMENT

Date: 04th June 2019

Description of work

To unload 1 unit machine from 1 x container, hoist up to 2nd floor by using platform and mobile/lorry crane

Machine Spec

1 unit DL45L CNC Lathe

Dim: 5810 x 2182 x 2374 mm

Wt: 13, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

10-11/06/2019   -    10:00 am

Location of Operation

#### 3 Joo Koon Circle

#### AME International Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x platform

1 set wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Hoist up platform together with machine to 2nd floor

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Jack up machine and insert rollers under machine bed

- Shift out machine from platform and transport to location

- Position machine to the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be dispose and clear

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175