# METHOD OF STATEMENT

Date: 07th Sep 2018

Description of work

1) To unload 1 unit machine from 1 x 20ft GP container, hoist up to 2nd floor by using platform and lorry crane

2) To shift out and hoist down 1 unit CNC Lathe from 2nd floor and load onto lorry chassis

Machine Spec

New Machine

GLS-1500 CNC Lathe

Dim: 2124 x 1523 x 1680 mm

Wt: 3, 000 kgs

Old Machine

GLS-150 CNC Lathe

Dim: 2432 x 1643 x 1727 mm

Wt: 3, 200 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

13/09/2018   -    09:30 am

Location of Operation

#### 39 Joo Koon Circle

#### Nutek Pte Ltd

Equipment Use

1 x 50 ton lorry crane

1 x lorry crane

1 x platform

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Hoist up platform together with machine to 2nd floor

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Insert pallet truck to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be dispose and clear

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175