# METHOD OF STATEMENT

Date: 16th Jan 2018

Description of work

To shift out 1 unit Bar Feeder and 1 unit machine from #01-13 to loading area, hoist up to 7th floor by using platform and mobile crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

24/01/2018   -    09:30 am

Location of Operation

#### Blk 1001 Jalan Bukit Merah #07-13

#### Petracarbon Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x platform

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Shift out machine and bar feeder to loading area

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Hoist up platform together with machine to 7th floor swing gate opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Insert pallet truck/rollers to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be dispose and clear

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175