# METHOD OF STATEMENT

Date: 29th Jan 2020

Description of work

To hoist up 1 unit bar feeder to 7th floor and hoist down 3 unit bar feeder from 7th floor using platform and mobile crane

Bar Feeder Spec

Dim: 480 x 30 x 150 cm

Wt: 300 kgs each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Time Of Operation

Location of Operation

#### Blk 3 A.M.K Industrial Pk 2A #07-01

#### JP Precision Engineering Pte Ltd

Equipment Use

1 x 70 ton mobile crane

1 x lorry crane

1 x platform

1 set wire/webbing sling

1 set shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated hoisting area

- Load bar feeder onto platform

- Secured bar feeder onto platform with strapping belt

- Hoist up platform together with bar feeder to 7th floor swing gate opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert rollers to bar feeder stand

- Shift out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be dispose and clear

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175