# METHOD OF STATEMENT

Date: 10th July 2017

Description of work

To hoist up 1 unit Conveyor and 1 unit Oven to 2nd floor by using platform and lorry crane

1 x Conveyor

Dim: 320 x 120 x 115 cm

Wt: 400 kgs

1 x Oven

Dim: 1m x 5.7m x 2.65m

Wt: 1,000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

12/07/2017   -    10:00 am

Location of Operation

#### 505 Yishun Industrial Pk A / PBA Building

#### Simplexity Pte Ltd

Equipment Use

1 x 45/50 ton lorry crane

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for the hoist

- Load conveyor/oven into platform

- Strap and secured load onto platform with strapping belt

- Hoist up platform together with machine to 2nd floor opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Insert pallet truck/rollers to conveyor/oven base

- Shift out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be dispose and clear

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175