# METHOD OF STATEMENT

Date: 18th Jan 2019

Description of work

1) To hoist up 9 unit bar feeder to 7th floor by using platform and mobile crane

2) To unload machine and accessories at loading bay, shift and position to 7th floor via cargo lift

Bar Feeder Spec

Dim: 380 x 47 x 102 cm

Wt: 200 kgs each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

23/01/2019

Time Of Operation

Estimate 11:00 am

Location of Operation

#### Blk 3 A.M.K Industrial Pk 2A #07-09

#### Rui Enterprise Pte Ltd

Equipment Use

1 x 50 ton mobile crane

3-4 x lorry crane

1 x platform

1 set wire/webbing sling

1 set shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Secured Bar Feeder onto platform with strapping belt

- Hoist up platform together with Bar Feeder to 7th floor swing gate opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert pallet truck/rollers to Bar Feeder base

- Shift out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be dispose and clear

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175