# METHOD OF STATEMENT

Date: 28th Aug 2024

Description of work

To hoist down 1 unit machine from 2nd floor shutter door opening of building by using platform and mobile crane

Machine Spec

1 unit Roders machine

Dim: 230 x 250 x 235 cm

Wt: 6, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

31/08/24

Time of Operation

09:45 am

Location of Operation

#### Simtech

#### 5 Clean Tech Loop, Clean Tech II Blk B

Equipment Use

1 x 50ton mobile crane

2 x 633 lorry loader

Rollers/pallet truck

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile crane at designated hoisting area

- Setting up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Secured sling to rigging point of platform

- Hoist up platform to 2nd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine to near platform

- Use of manual pulley to slowly pull in machine into platform

- Jack up machine, pull out rollers and inset woodblock under machine bed

- Secured machine onto platform with strapping belt

- Mobile crane to hoist down machine together with platform to the ground

- Load up onto lorry chassis

- Transport and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175