# METHOD OF STATEMENT

Date: 27th Feb 2024

Description of work

To hoist down 1 unit machine from 3rd floor of building by using platform and lorry crane

Machine Spec

Assembly Machine

Dim: 200 x 156 x 150 cm

Wt: 950 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

05/03/24

Time of Operation

10:00 am

Location of Operation

#### Blk 2023 Bukit Batok St 23 #03-92/96

#### Pocket Technology Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 x platform

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Setting up of crane at site location

- Hoist up platform to 3rd floor

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building parapet wall

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Use of customer reach truck to fork up machine

- Load up machine onto platform

- Secured machine onto platform with strapping belt

- Hoist down machine together with platform to the ground

- Load up onto lorry chassis

- Transport and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175