# METHOD OF STATEMENT

Date: 19th Sep 2023

Description of work

To hoist up 1 machine to 3rd floor of building by using scissor platform, platform and lorry crane

Machine Spec

1 unit AR45 Wire Cut

Dim: 120 x 170 x 180 cm

Wt: 1, 700 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29/11/23 (Wed)

Time of Operation

09:45 am

Location of Operation

#### Blk 3007 Ubi Rd 1 #03-448

#### AARON WIRECUT

Equipment Use

2 x 633 lorry loader

Pallet truck/rollers

1 x working platform

1 x scissor platform

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of 2 x lorry crane at designated hoisting area

- Setting up of crane at site location

- Hoist up platform to 3rd floor

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building parapet wall

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Tied back platform to competent point and secured with 1 x lorry crane

- 1 x lorry crane to hoist up machine to 3rd floor

- Lower down machine onto platform

- Insert pallet truck under machine bed

- Shift out machine from platform and load onto scissor platform

- Lower down scissor platform

- Shift out machine from scissor platform

- Transport machine into customer premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175