# METHOD OF STATEMENT

Date: 23rd Dec 2016

Description of work

1) To shift out and hoist down 3/4 unit machine from 3rd floor by using platform and lorry crane, load onto lorry chassis and transport to new location

2)  Shift out 9 unit small machine and accessories from 3rd floor via cargo lift and load onto lorry chassis

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

29-30/12/2016   -   09:30 am (shift out via cargo lift)

15:00 pm (hoisting through swing gate)

Location of Operation

#### Blk 1 Kaki Bukit Ave 3 #03-02

(Fifth Engineering)

Equipment Use

1 x 50 ton lorry crane

1 x 45 ton lorry crane

Lorry crane

1 x platform

Wire Sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris, if any will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade operation area, assignment of traffic controller to control movement within the vicinity                                                            All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Barricade operation area with safety cones and red/white tape

- Hoist up platform to 3rd floor opening

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production room into platform

- 50 ton lorry crane to hoist down machine from platform

- Lower down machine and load onto lorry chassis

- Secured machine with belt prior to moving out of customer premise and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175