# METHOD OF STATEMENT

Date: 22 September 2015

Description of work

To shift out and hoist down 1 unit EMV600 CNBC Milling machine from 5th floor, load onto lorry chassis and transfer to new location

To move in 1 unit new machine to 5th floor by using cargo lift

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

23/09/2015 - 09:30am

Location of Operation

#### Blk 1 Kaki Bukit Ave 3 #05-05

(QE Engineering Pte Ltd)

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x platform

Rollers / pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# Job Scope

- 50 ton mobile crane to park at designated hoisting area for the lift

- Barricade operation area

- Hoist up platform to 5th level opening

- Tagline will be use to secure and control the swing motion of platform

- Stabilize platform and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift machine into platform

- Secured machine with strapping belt

- Hoist down machine platform together with machine and load onto lorry chassis

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175