# METHOD OF STATEMENT

Date: 28th June 2016

Description of work

To hoist up 1 unit machine to 2nd floor by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

13/07/2016 – 10:00 am

Location of Operation

#### Blk 1002 Jalan Bukit Merah #02-01/03

Maxstech Technologies (S) Pte Ltd

Equipment Use

1 x 50 ton mobile/lorry crane

1 x 35 ton lorry crane

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Barricade operation area

- Hoist up platform to 2nd level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 35 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- 50 ton mobile/lorry crane to hoist up machine to 2nd level opening,

- Lower down machine onto platform

- Insert rollers/pallet truck to machine base, shift out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175