# METHOD OF STATEMENT

Date: 02nd June 2021

Description of work

1) To shift out 4 unit machine from ground floor at Blk 7 Kallang Place, deliver to Blk 16 Kallang Place and hoist up to 3rd floor

2) To hoist down 3 unit machine from Blk 16 Kallang Place 3rd floor, load onto lorry chassis, deliver to Blk 7 Kallang Place and unload at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

02/06/21 – 02/08/21

Time of Operation

09:30 am

Location of Operation

#### Blk 7 Kallang Place #01-04

#### Blk 16 Kallang Place # 03-14/15

Li Zheung Precision Engineering

Equipment Use

1 x 50 ton mobile crane

1 x 45/50 ton lorry loader

3 x lorry crane

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Hoist up platform to 3rd floor swing gate opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45/50 ton lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 3rd floor

- Lower down machine slowly onto platform

- Insert rollers/pallet truck to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175