# METHOD OF STATEMENT

Date: 09th Sep 2022

Description of work

To hoist up 6 unit machine to 2nd floor through swing gate opening by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

01/10/22 (Saturday job)

Time of Operation

10:30 am

Location of Operation

#### Blk 16 Kallang Place # 02-15

Inzign Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 45 ton lorry crane

2-3 x lorry

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Tied back platform to a competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 2nd floor

- Lower down machine slowly onto platform

- Insert rollers under machine base

- Shift out machine from platform

- Transport and position machine into tenant premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175