# METHOD OF STATEMENT

Date: 23rd Feb 2018

Description of work

To hoist up 1 unit Focaseiki PM1100 CNC machine to 2nd floor by using platform and lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

03/03/2018   –    09:45 am

Location of Operation

#### Blk 20 Woodlands Link #02-05

Accelerated Precision

Equipment Use

1 x 50 ton lorry crane

1 x 35/45 ton lorry crane

1 x platform

Rollers/pallet truck

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety net/red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 2nd level opening

- Stabilize platform and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 35/45 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift machine into platform

- 50 ton lorry crane to hoist down machine and load onto lorry chassis

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175