# METHOD OF STATEMENT

Date: 02nd Jan 2019

Description of work

1) To hoist down 2 unit machine from 2nd floor by using platform and lorry crane

2) To hoist up 1 unit machine to 2nd floor by using platform and lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

12/01/2019   –   09:30 am

Location of Operation

#### Blk 26 Kallang Place #02-17

Inzign Pte Ltd

Equipment Use

1 x lorry crane

1 x platform

Wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for the hoist

- Load machine onto platform

- Strap and secured machine onto platform with strapping belt

- Hoist up platform together with machine to 2nd floor opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert pallet truck to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be dispose and clear

- Job complete

Remark:

To hack 1 layer parapet wall, dismantle sun breaker for machine to move in and make good upon job completion

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175