# METHOD OF STATEMENT

Date: 10th July 2018

Description of work

To unload 4 cases of 2 unit machine from 1 x 40ft FR and 1 x 40ft GP container, uncrate, hoist up and load onto platform, shift and position into customer premise at ground

Machine Spec

2 unit Mazak VCN-530C Machining Center

Dim: 3150 x 2850 x 2800 mm

Wt: 7,000 kgs each

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

24-28/07/2018   -   09:30 AM

Location of Operation

#### Blk 30 Loyang Way #01-10

Inter-Tooling Engineering Pte Ltd

Equipment Use

2 x lorry crane

1 x platform - to lay out for machine pathway

Heavy duty rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones/red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for the lifting

- Lay out platform for machine pathway

- Hoist down cases from FR container and load onto ground

- Uncrate machine case

- Hoist up and detach machine from base

- Load machine onto platform

- Insert rollers under machine base

- Shift out from platform and position into premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

To hack 1 layer internal low wall, dismantle window, window frame and make good upon job completion

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175