# METHOD OF STATEMENT

Date: 28th March 2016

Description of work

To hoist down 7 unit CNC machine from 2nd floor by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

01-02/04/2016 – 09:30 am

(2 day job)

Location of Operation

#### Blk 30 Loyang Way #02-22/23

(Freudenberg Oil & Gas Technologies)

Equipment Use

1 x 50 ton mobile

1 x 50 ton lorry crane

Lorry crane

1 x low-loader

1 x platform

Wire Sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade operation area, assignment of traffic controller to control movement within the vicinity                                                            All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Barricade operation area with safety cones and red/white tape

- Hoist up platform to 2nd floor opening

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production room into platform

- 50 ton mobile crane to hoist down machine from 2nd floor

- Lower down machine and load onto low-bed/lorry chassis

- Secured machine with belt prior to moving out of customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175