# METHOD OF STATEMENT

Date: 22nd Feb 2018

Description of work

To unload/hoist down 2 unit Enshu GE480 CNC machine from 1 x 40ft FR container, uncrate, hoist up and load onto platform, shift and position into customer premise at ground

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

02-03/03/2018   -   09:30 AM

Location of Operation

#### Blk 3014A Ubi Rd 1 #01-02/03

Progress Engineering Services Pte Ltd

Equipment Use

1 x 50 ton mobile/lorry crane

1 x lorry crane

1 x platform - to lay out for machine pathway

Heavy duty rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones/red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for the lifting

- Lay out platform for machine pathway

- Hoist down cases from FR container and load onto ground

- Uncrate machine case

- Hoist up and detach machine from base

- Load machine onto platform

- Insert rollers under machine base

- Shift out from platform and position into tenant premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

To dismantle sun breaker/pillar and fix back immediately upon job completion

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175