# METHOD OF STATEMENT

Date: 11th Aug 2017

Description of work

1) To shift out 1 unit Feeler QM32 machine from production and load onto forwarder lorry

2) To hoist down 1 unit CV 14A CNC machine from lorry chassis, load onto platform, shift and position into customer premise at ground

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

21/08/2017   -   09:30 AM

Location of Operation

#### Blk 3014A Ubi Rd 1 #01-02

Progress Engineering Services Pte Ltd

Equipment Use

2 x lorry crane

1 x platform - to lay out for machine pathway

Heavy duty rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated area for the lift

- Barricade operation area with safety cones, red white tape

- Lay out platform for machine pathway

- Hoist up machine from lorry chassis and load onto platform

- Insert rollers under machine base

- Shift out from platform and position into tenant premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175