# METHOD OF STATEMENT

Date: 17th Aug 2018

Description of work

1) To hoist up one unit Bar Feeder to 4th floor by using lorry crane

2) To unload at loading bay 4 unit machine, rack, cabinet and estimate 30 pallet from lorry chassis, shift and position to 4th floor via cargo lift

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBA

Time Of Operation

11:00 am

Location of Operation

#### Blk 3014A Ubi Rd 1 #04-11

Henko Machine Tools Pte Ltd

Equipment Use

1 x 45 ton mobile crane

2 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Insert webbing sling onto bar feeder hoisting point and secured onto crane hook

- Hoist up bar feeder to 4th floor

- Unload bar feeder at 4th floor through swing gate opening

- Insert rollers under bar feeder stand

- Shift and position into premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175