# METHOD OF STATEMENT

Date: 19th July 2016

Description of work

1) To unload 13 unit machine from lorry chassis at loading bay, lift up using lorry crane, insert pallet truck to machine base, shift and position to 2nd floor via cargo lift

2) To unload 7 unit CNC machine from lorry chassis, hoist up to 2nd floor, lower down onto platform, insert rollers/pallet truck to machine base, shift and position into customer premise

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

29/07/2016 – 11:00 am – unload, shift and position machine to 2nd floor via cargo lift

30/07/2016 – 11:00 am -  Hoist up to machine to 2nd floor by using platform and mobile/lorry crane

Location of Operation

#### Blk 3015A Ubi Rd 1 #02-02

(Cosmag Plastic Mould Pte Ltd)

Equipment Use

1 x 50 ton mobile

1 x 50 ton lorry crane

Lorry crane

1 x platform

Wire Sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement within the vicinity                                                            All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness, etc..)

Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 2nd floor opening

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secured front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 2nd floor

- Lower down machine and onto platform

- Insert pallet truck/rollers to machine base

- Shift out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175