# METHOD OF STATEMENT

Date: 03rd Dec 2018

Description of work

1) To hoist up 4 unit machine to 3rd floor by using platform and mobile/lorry crane

2) To unload at loading bay 18 unit small machine and accessories from lorry chassis, shift and position to 3rd floor via cargo lift

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

12/12/2018   -   To deliver and unload small machine and accessories to 3rd floor via cargo lift

13/12/2018   -   To hoist up 4 unit machine to 3rd floor using platform and mobile/lorry crane

14/12/2018   -   To deliver balance of machine and accessories

Time Of Operation

10:00 am   -   19:00 pm

Location of Operation

#### Blk 3015A Ubi Rd 1 #03-06

Ching Yi Technology Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50/45 ton lorry crane

2-3 x lorry

1 x platform

Pallet truck/rollers

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 3rd floor swing gate opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 50/45 ton lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 3rd floor

- Lower down machine slowly onto platform

- Insert rollers/pallet truck onto machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd / Tel: 62824175