# METHOD OF STATEMENT

Date: 29 Feb 2016

Description of work

To hoist down 1 unit equipment from 4th floor by using platform and mobile crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

To be advised

Location of Operation

Equipment Use

1 x 100/160 ton mobile crane

1 x platform

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness,  etc…) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated location for the hoist

- Hoist up platform to 4th level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use of strong belt to secure front left and right of platform to prevent platform swing outward

- Use manual lifter to lift up equipment and level with platform

- Shift and load equipment onto platform

- Strap and secured equipment onto platform with strapping belt

- Hoist down platform together with equipment, load onto lorry chassis and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be dispose and clear

- Job complete

Best Regards

Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175