# METHOD OF STATEMENT

Date: 30th Oct 2017

Description of work

To hoist down 1 unit Sakutai ST-102 Finishing machine and 2 unit accessories from 6th floor by using platform and mobile crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

02-09/11/17   -   14:00 PM

Location of Operation

#### Blk 3015A Ubi Rd 1 #06-02

Gold Kwang Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x platform

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be

involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness,  etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated location for the hoist

- Hoist up platform to 6th level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use of strong belt to secure front left and right of platform to prevent platform swing outward

- Shift and load machine into platform

- Strap and secured machine onto platform with strapping belt

- Hoist down platform together with machine

- Hoist up and transfer machine from platform to Malaysia lorry

- Secured machine with belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be dispose and clear

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175