# METHOD OF STATEMENT

Date: 27 July 2015

Description of work

To hoist up 1 unit machine at loading bay, insert pallet truck, shift and position into customer premise by using cargo lift

Risk Assessment

A copy of Risk Assessment will be attach.

Date / Time of Operation

31/07/2015 - 09:30am

Location of Operation

#### Blk 3015A Ubi Rd 1 #07-06

Rexxson Pte Ltd

Equipment Use

1 x lorry crane

Heavy duty rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc…)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated area for the lift

- Barricade operation area with safety cones

- Crane to hoist up machine from loading bay

- Insert pallet truck under machine base

- Push out machine form lorry chassis

- Use cargo lift to shift machine into customer premise at 7th floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175