# METHOD OF STATEMENT

Date: 02nd July 019

Description of work

To hoist up 1 unit machine to 5th floor through swing gate opening by using platform and mobile/lorry crane

Machine Spec

1 unit BMC-500 CNC Lathe

Dim: 340 x 227 x 182 cm

Wt: 3, 275 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

02/09/2019   –   09:45 am

Location of Operation

#### Blk 3016 Bedok North Ave 4 #05-19

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

Pallet truck/rollers

1 x platform

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 5th level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 50 ton mobile to hoist up machine to 5th level opening

- Lower down machine onto platform

- Insert rollers/pallet truck under machine bed

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175