# METHOD OF STATEMENT

Date: 17 Dec 2015

Description of work

To hoist up 3 unit CNC machine to 2nd floor by using platform and lorry crane

Working Area

Parking lot no: 10-17

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

23/12/2015 – 09:30 am

Location of Operation

#### Blk 3023 Ubi Rd 3 #02-04

Sing Geyi Precision Engineering

Equipment Use

1 x 50 ton lorry crane

1 x 45 ton lorry crane

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc...) Lifting machine and lifting gear with M.O.M cert.

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Barricade operation area

- Hoist up platform to 2nd level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- 50 ton lorry crane to hoist up machine to 2nd level and lower down slowly onto platform

- Insert rollers/pallet truck to machine base

- Shift machine out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175