# METHOD OF STATEMENT

Date: 22nd Aug 2016

Description of work

To shift out 3 unit Injection Molding machine from 6th floor, load onto platform and hoist down  to 5th floor

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

27/08/2016 – 09:00 am

Location of Operation

#### Blk 4 Kaki Bukit Ave 1 / From #06-07 - #05-01

Moveon Technologies Pte Ltd

Equipment Use

1 x 70 ton mobile crane

1 x lorry crane

1 x platform

Rollers / pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# Job Scope

- 70 ton mobile crane to park at designated hoisting area for the lift

- Hoist up platform to 6th level opening

- Stabilize platform and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift machine into platform

- Secured machine with strapping belt

- Hoist down platform together with machine to 5th level opening

- Stabilize platform and ensure it is fully rested on the opening slab

- Move out machine from platform, shift and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175