# METHOD OF STATEMENT

Date: 23rd July 2019

Description of work

To unload/hoist down 1 unit machine from lorry chassis, load onto platform, shift and position into customer premise at ground

Machine Spec

QT 250MY CNC Lathe

Dim: 290 x 293 x 265 cm

Wt: 6, 650 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

25/07/2019

Time Of Operation

After 5 pm

Location of Operation

#### Blk 4010 A.M.K Tech Park 1 #01-04/05/06

3 Star Engineering & Trading

Equipment Use

2 x lorry crane

1 x platform - to lay out for machine pathway

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated loading area for the lift

- To lay out platform for machine pathway

- Hoist down machine from lorry chassis and load onto platform

- Insert rollers under machine base

- Shift out machine from platform into customer premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175