# METHOD OF STATEMENT

Date: 07th Nov 2018

Description of work

1) To hoist up 5 units machine to 2nd floor by using platform and mobile/lorry crane

2) To unload 15 unit machine, 1 lots cabinets, table, racking and materials at loading bay, shift and position to 2nd floor via cargo lift

Machine Spec

Refer to attached

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

16/11/2018   -   To deliver small machine and accessories to 2nd floor via cargo lift

17/11/2018   -   To hoist up machineries to 2nd floor using platform and mobile/lorry crane

Time of Operation

11:00

Location of Operation

#### Blk 4010 AMK Ave 5 #02-10/11

LHY Engineering (S) Pte Ltd

Equipment Use

1 x 50 ton mobile/lorry crane

1 x 35/45 ton lorry crane

3-4 x lorry

1 x platform

Wire/webbing sling

2 x Retractable Life Line

Safety harness

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 2nd floor opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 35/45 ton lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 50 ton mobile/lorry crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert rollers/pallet truck to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175