# METHOD OF STATEMENT

Date: 01st Sep 2025

Description of work

1) To unload/hoist down 2 units machine with accessories from lorry chassis, load onto platform, shift and position into customer premise at ground

2) To shift out 1 unit machine from #01-10 and move into #01-03

Machine Spec

To unit #01-03

2 units VCN-430AL

Dim: 2600 x 2100 mm

Wt: 5, 000 kg each

From unit #01-10 to #01-03

1 unit DMU 50

Dim: 220 x 218 x 250 cm

Wt: 4, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

27/09/25 (Sat)

Time of Operation

09:45 am

Location of Operation

Whits Technologies Pte Ltd

#### Blk 4012 A.M.K Ave 10 #01-03, #01-10, Tech Park 1

Equipment Use

3 x lorry loader

1 x platform - to lay out in front of tenant unit for machine pathway

Rollers/pallet truck

Wire/webbing sling

Shackle

Woodblock

Hand jack

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area for the lifting work

- To lay out platform in front of tenant unit for machine pathway

- To hoist down case from lorry chassis to the ground

- Uncrate case and put aside

- Insert sling to crane hook and secured to rigging point of machine

- Hoist up machine and detach machine from base

- Swing crane and lower down machine onto platform

- Insert rollers/pallet truck under machine bed

- Shift out machine from platform

- Transport machine into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175