# METHOD OF STATEMENT

Date: 15th Sep 2021

Description of work

1) To shift out 2 unit machine from customer premise, hoist up onto lorry chassis and deliver to other location

2) To unload 2 unit machine, uncrate, shift and position into tenant premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Schedule of Operation

07/10/21   -   Hacking of wall after 6 pm onwards

08/10/21   -   Move out 2 unit PM860 and hoist up onto lorry

09/10/21   -   Move in 2 unit Mazak VCN530

Time of Operation

09:45 am

Location of Operation

#### Blk 4012 AMK Ave 10 #01-10

WHITS Technologies

Equipment Use

2 x lorry crane

1 x trailer

1 x platform - to lay out for machine pathway

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Deploy lorry loader at designated unloading area

- Hoist down platform from lorry chassis and lay out in front tenant door for machine pathway

- Hoist down machine from lorry chassis and load onto platform

- Insert pallet truck/rollers under machine base

- Shift out machine from platform

- Transport and position machine into tenant premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175