# METHOD OF STATEMENT

Date: 16th Sep 2022

Description of work

1) To shift out 2 unit machine from tenant premise at ground floor onto platform, hoist up and load onto lorry chassis

2) To hoist down 2 unit machine from low loader chassis, load onto platform, shift and position into customer premise at ground

Machine Spec

Mazak Vertical Machining Center / VCN-530L

Dim: 319 x 308 x 304 cm

Wt: 6, 890 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

22/09/22 (to hack internal wall after 6pm)

23/09/22 (to shift out 2 unit machine and load onto lorry after 1pm)

24/09/22 (to move in 2 unit new machine)

Time of Operation

09:45 am

Location of Operation

#### Blk 4012 A.M.K Tech Park 1 #01-10

Whits Technologies Pte Ltd

Equipment Use

1 x 633 FALFINGER lorry loader

2 x lorry

1 x low loader

1 x platform - to lay out for machine pathway

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated loading area for the lift

- To lay out platform for machine pathway

- Reverse low loader to near crane

- Hoist up machine from low loader chassis

- Swing crane and load machine onto platform

- Insert rollers under machine base

- Shift out machine from platform into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175