# METHOD OF STATEMENT

Date: 29th May 2017

Description of work

1) To hoist down 1 unit DMG CNC machine from 3rd floor and load onto lorry chassis

2) To hoist up 1 unit HEM 700U CNC machine to 3rd floor, shift and position into customer premise

Machine Spec

DMG CNC machine

Dim: 220 x 220 x 220 cm

Wt: 5, 000 kgs

HEM 700U CNC

Dim: 229 x 360 x 254 cm

Wt: 8, 400 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

01/06/2017   -   09:30 am

Location of Operation

#### Blk 5 A.M.K Industrial Pk 2A #03-10

Illuminate Precision Engineering Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 x platform

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 3rd level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 3rd floor

- Lower down machine slowly onto platform

- Insert rollers/pallet truck to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175