# METHOD OF STATEMENT

Date: 15th Jan 2019

Description of work

To shift out machineries from production room into platform, using lorry crane to hoist up and load onto lorry chassis

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

17-18/01/2018

Time Of Operation

09:30 am

Location of Operation

#### Blk 5000 AMK Ave 5 #01-04

Micron Tech Engineering

Equipment Use

3-4 x lorry crane

1 x platform - to lay out for machine pathway

Rollers/pallet truck

1 set wire sling

1 set shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lay out platform in front of tenant door for machine pathway

- Barricade operation area with safety cones, red/white tape and lifting in operation signage

- Shift out machine from production room

- Load machine into platform

- Lorry crane to hoist up machine

- Load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of  premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

Contractor to dismantle wind breaker and fix back immediately upon job completion

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175