# METHOD OF STATEMENT

Date: 24th May 2017

Description of work

To shift out 1 unit Twinhorn CNC Milling, 1 unit Good Way CNC Lathe and 2 unit Milling machine from ground floor, load onto lorry chassis and deliver to new location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

01/06/2017   -   09:30am

Location of Operation

#### Blk 5004 A.M.K Ave 5 #01-09

Common Alliance Engineering Pte Ltd

Equipment Use

2 x lorry crane

Rollers / pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry crane to loading bay

- Flush up lorry crane to level with loading bay

- Shift out machine from production to loading bay

- Load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of  premise and transport to customer new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

To hack 1 layer internal wall, dismantle window frame, cut corridor railing and will make good after machine move out

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175