# METHOD OF STATEMENT

Date: 27th June 2017

Description of work

To hoist out 1 Set of Machinery from Block 5014 Ang Mo Kio Ave 5, Techplace II #05-03. Machine size: (Dim: 200 x 185 x 230cm) within platform size.

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

1st July 2017 (Saturday)

Location of Operation

#### Blk 5014 A.M.K Ave 5 #05-03

Equipment Use

1 x 50 ton Mobile crane

1 x 45 ton Lorry Crane

1 x Pallet truck

2 x Retractable life line

4 x Safety Harness

1 x Platform

Wire sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area.

- Jack Machine up using Toe Jack or Heavy duty Crowbar.

- Insert pallet truck to machine base, shift out from platform and position into customer premise.

- Ensure all Platform railing is up in place.

- Ensure Anchor Point before opening of swing gate.

- Hoist up platform to 5th floor opening.

- Stabilize platform effectively and ensure it is fully rested on the opening slab.

- Tied back platform to a competent point and secured using 45 ton lorry crane.

- Use strong belt to secure front left and right of platform to prevent platform swing outward.

- 50 ton mobile crane to hoist down machine from 5th floor opening.

- Lower down machine onto lorry.

- Strap down Machine onto Lorry.

- Leave premises with guidance.

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose.

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175