# METHOD OF STATEMENT

Date: 19th Mar 2018

Description of work

To shift out 6 unit machine, 10 set locker and 2 set of Rack from production, hoist up and load onto Malaysia lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

22/03/2018   -    09:45 am

Location of Operation

#### Blk 55 Ayer Rajah Crescent #01-07/09

Sky Engineering Pte Ltd

Equipment Use

2 x lorry crane

2 x Malaysia lorry

1 x platform - to lay out for machine pathway

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated location for the lift

- Lay out platform for machine pathway

- Shift out machine from production room into platform

- Hoist up machine and load onto Malaysia lorry

- Secured machine with strapping belt prior to moving out of  premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175