# METHOD OF STATEMENT

Date: 21st June 2021

Description of work

To hoist down 1 unit old machine from 2nd floor and hoist up 1 unit new machine to 2nd floor through swing gate opening by using platform and lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

03-24/07/2021

Time of Operation

09:45 am

Location of Operation

#### Blk 7 Kallang Place #02-06

Lek Hung Moulding Pte Ltd

Equipment Use

1 x 50 ton lorry crane

1 x 45 ton lorry crane

1 x lorry crane

1 x platform

Wire sling/shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry carne at designated hoisting area for the lift

- Hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- 50 ton lorry crane to hoist up machine to 2nd floor

- Lower down machine slowly onto platform

- Insert rollers/pallet truck to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Remark

To dismantle sun breaker and make good upon job completion

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175