# METHOD OF STATEMENT

Date: 13th Nov 2017

Description of work

To shift out and hoist down Shimada Machine from 7th floor, load onto wooden base for packing, after packing put aside or load onto container/Malaysia lorry

Machine Spec

Shimada Machine (dismantle into parts)

Dim: 500 x 181 x 225 cm

Wt: 4, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date/Time of Operation

As per required schedule

Location of Operation

#### Blk 8 Kaki Bukit Ave 1

HGST Singapore Pte Ltd

Equipment Use

1 x 70 ton mobile crane

1 x 50 ton mobile crane

1 x lorry crane

1 x platform

Roller/pallet truck

Wire/webbing sling

1 x 4.5 ton forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified Safety Supervisor, Lifting Supervisor, Rigger, Signalman and Mobile/Lorry Crane Operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness,  etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- 2 x mobile crane to park at designated location for the hoist

- Hoist up platform to 7th floor opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 50 ton mobile crane

- Use of strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production room and load onto platform

- Insert webbing sling to machine hoisting point

- 70 ton mobile crane to hoist down machine

- Load machine onto wooden base

- Pack machine into wooden case

- After packing, put case aside or load onto container/Malaysia lorry

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175