# METHOD OF STATEMENT

Date: 14 May 2015

Description of work

30/05/2015-To unload 10 packages of machine parts from 1 x 40ft O/T container, uncrate, shift and position into customer premise via cargo lift

31/05/2015- To unload 3 packages of 1 unit Bysun Fiber 3015 Laser Cutting machine from

1 x 40ft O/T container

Main machine - Hoist up to 3rd floor

Machine parts - Via cargo lift

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

30-31/05/2015 - 10:00 am

Location of Operation

#### Blk 8 Pandan Crescent #03-03

James Walker Asia Pacific Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 40/50 ton lorry crane

1 x platform

1 x forklift

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorm and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, and mobile/lorry crane will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervised by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated location for the lift

- Hoist up machine from O/T container

- Load machine to the ground

- Uncrate machine case

- Hoist up platform to 3rd level opening

- Stabilize platform effectively and to ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 50 ton lorry crane

- 50 ton mobile crane to hoist up machine to 3rd floor

- Lower down machine onto platform

- Insert rollers to machine base

- Shift out machine from platform and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175